PROGRESSIVE SCG PRODUCT SPECIFICATIONS                                                  
APRIL 2016
Progressive Sports Construction Group, 411 South Pearl Street, Spiceland, Indiana U.S.A. 47385-0425

TEL:  877-413-0957;   FAX:   866-637-5611;   WEB SITE:  www.progressive scg.com

SECTION 11 66 53.13PRIVATE 

GYMNASIUM PRACTICE CAGES

***** Progressive Sports Construction Group manufactures practice cages which can be electrically lowered and raised from gymnasium ceilings to provide interior spaces for hitting baseballs, softballs, golf balls, and tennis balls; kicking footballs and soccer balls; and other athletic practice activities.

PART 1 - GENERAL
1.1
SUMMARY


A.
Section includes: Electrically operated net gymnasium practice cage lowered from ceiling to provide interior space for indoor athletic practice activities.



B.
Related sections:

***** List other specification sections dealing with work directly related to this section such as the following.*****



1.
Section [_____] - [__________]: Structural steel framing to support gymnasium practice cage.




2.
Section 16[___] - [__________]: Electrical supply, conduit, and wiring for motorized gymnasium practice cage.

1.2

seq level1 \h \r0 

seq level2 \h \r0 

seq level3 \h \r0 
SUBMITTALS


A.
Submit in accordance with Section 01 33 00 [01330] - Submittal Procedures:



1.
List of proposed products and product data.




2.
Loads to be transmitted to building structural members and requirements for supplementary bracing and structural support members.




3.
Shop drawings showing layout, elevations, dimensions, fabrication details, method of attachment, and electrical wiring diagrams.




4.
Samples of fabric net.




5.
Manufacturer's installation and maintenance instructions.

1.3

seq level1 \h \r0 

seq level2 \h \r0 

seq level3 \h \r0 
QUALITY ASSURANCE


A.
Source limitation: All components including frame, netting, suspension system, electric winches, and controls for practice cage shall be products of a single manufacturer.

1.4

seq level1 \h \r0 

seq level2 \h \r0 
DELIVERY, STORAGE, AND HANDLING
*****In order to reduce potential damage to cage and netting, it is desirable to deliver and install practice cage after gymnasium flooring and other interior construction is essentially complete.*****


A.
Do not deliver practice cage until building is enclosed and, other construction within gymnasium is substantially complete.

PARTS 2 - PRODUCTS
2.1

seq level1 \h \r0 

seq level2 \h \r0 
ACCEPTABLE MANUFACTURERS


A.
Progressive Sports Construction Group, 411 South Pearl Street, Spiceland, Indiana 47385-0425; 877-413-0957.



B.
Manufacturers of equivalent products submitted and approved in accordance with Section 01630 - Product Substitution Procedures.

2.2

seq level1 \h \r0 

seq level2 \h \r0 
GYMNASIUM PRACTICE CAGE


A.
Type: Electrically operated, gymnasium practice cage including motor, cables, controls, clamps for attachment to building structure, threaded rod supports, and other components required for complete functional installation; Practice Cage as manufactured by Progressive Sports Construction Group.

*****Use below paragraph for Center Lifting and Side Lifting Practice Cage.*****


B.
Operation: Cage frame moved up and down by [1/8 inch] [3.2 mm] 7/19 galvanized aircraft cables wound onto overhead rotating drive pipe operated by electrical motor. For storage, cage is lowered, mesh gathered on top of frame, and frame raised to ceiling.

*****Use below paragraph for Bottom Lifting Practice Cage.*****


B.
Operation: Netting and cage frame moved up and down by [1/8 inch] [3.2 mm] 7/19 galvanized aircraft cables wound onto overhead rotating drive pipe operated by electrical motor. Cables pass through d-rings on nylon belts allowing belt to accordion fold. Lift cables pass through links attached to netting at approximately [12 inches] [305 mm] c about [12 inches] [305 mm] causing netting to lift to the cage frame as the entire assembly is lifted to the ceiling.
*****Standard size of Progressive Sports Construction Group Practice Cage is 10 feet (3 m) high by 12 feet (3.7 m) wide by 70 feet (21 m) long. Other sizes can be custom fabricated.*****


C.
Configuration: Rectangular shape [10 feet] [3 m] [_____] high by [12 feet] [3.7 m] [_____] wide by [70 feet] [21 m] [_____] long.



D.
Frame: Constructed of [1-5/8 inches] [41 mm] diameter steel tubing with [0.109 inch] [3 mm] wall thickness. Assemble frame with malleable iron galvanized fittings with case hardened set screws.



E.
Operating mechanism: Drive pipe mechanism shall consist of a 3/4 HP, 110VAC, 60 cycle, single-phase, reversible capacitor with built-in thermal overload protection. The motor and load holding worm gear reducer shall provide speed reduction in the winch. Remote control operation with integral limit switches to control the upper and lower limit of cage travel.


F.
Attachment: Attach to structural support with beam clamps, hanger brackets, and [1/2 inch] [13 mm] diameter threaded rods. Attach at [10 feet] [3 m] centers.



G.
Hoist lines: [1/8 inch] [3 mm] diameter steel galvanized cable with 2,000 pounds minimum breaking strength. Space lines at approximately [10 feet] [3 m].
*****Use below paragraph for Center Lifting Practice Cage.*****


H.
Drive pipe is located at center of practice cage. Lift cables are attached 2/0 passing link chain that terminates at each side of the cage frame.
*****Use below paragraph for Side Lifting Practice Cage.*****


H.
Drive pipe is located at one side of practice cage and there shall swivel pulley assemblies to guide lifting cable to the other side of cage. Lift cables are attached to each side of the cage frame.

*****Use below paragraph for Bottom Lifting Practice Cage.*****


H.
Drive pipe is located at one side of practice cage and there shall swivel pulley assemblies to guide lifting cable to the other side of cage. Lift cables pass through guides at the cage frame then terminate at cable weights that are suspended at about [12 inches] [305 mm] above playing surface.
2.3

seq level1 \h \r0 

seq level2 \h \r0 
NETTING


A.
Type: [3/4 inch] [19 mm] square, knotless, [White (Standard)], [Black] nylon mesh.



B.
Material: #252 knotless nylon with 95 pounds breaking strength.



C.
Minimum weight: 0.05 pounds PSF.



D.
Perimeter of netting sections: Sewn with [3/4 inch] [19 mm] poly tape binding.



E.
Provide zippers at two corners for access to cage.



F.
Size of netting to be sufficient to allow [12 inches] [305 mm] of material to rest on floor in use positon.

2.4

seq level1 \h \r0 

seq level2 \h \r0 
PRACTICE CAGE SAFETY DEVICE

***** Include the following paragraphs to include Curtain Lok safety device *****

*****************************************************************************

A. Provide Draper Model 504301 Curtain Lok safety device. Curtain Lok to be directly speed sensitive to automatically lock divider curtain in position at any time during storage or operation. In the event of an over-speed situation (greater than 1.5 feet per second) caused by malfunction of the hoisting apparatus, whether sudden or gradual, device will immediately activate. Curtain Lok work regardless of direction of rotation and automatically resets when load is reversed or removed.

2.5seq level1 \h \r0 

seq level2 \h \r0 
CONTROLS

***** Include the following paragraph for standard key switch operation.*****


A.
Provide key lock, 3-position, momentary contact wall control switch to lower, raise, and stop gymnasium practice cage. Provide with stainless steel cover plate.

*****Include the following paragraphs for practice cages with EZ-Pad Plus Group Control System.*****

A. Provide EZ-Pad Plus Group Controller capable of controlling Electric Divider Curtains, Electric Height Adjusters, Basketball Backstops with Electric Winches, Wrestling Mat Lifters and other Auxiliary Devices from a centralized location. Group Control Systems is fully field programmable and supports individual device operation as well as operation of items of part of one or more group of up to 8 devices.
B. Devices are controlled via 120V relay/controller panels that may be mounted at remote location(s.) Each relay panel includes 18 relays configured in eight directional control pairs and two on-off individual relays. As many as (16) Relay Boxes may be networked to allow up to (128) directional gym equipment devices and up to (32) on-off auxiliary items to be operated. Relay Boxes shall distribute power to the appropriate devices as a result of commands entered at the Touch Pad. Relay Boxes shall be dual powered and sized to accept two 120 volt, 30 amp, single-phase circuits.

C. User interacts via low voltage, momentary contact, Touch Pad (12V) to communicate with the Relay/Controller enclosure. Touch pad should be mounted at such a position in the gymnasium as to allow a clear line of sight to all the equipment under the control of the system. Touch Pad shall be flush mounted and fit into a standard 8-5/8" x 4-1/2" x 2-1/2” electrical box (RACO Box #942 or equivalent). Touch Pads to have password protection to prevent usage by unauthorized personnel. Password protection is programmable and is multi-level with a 6-digit user’s security code as well as a separate 6-digit supervisor’s code. User code allows for operation of individual devices and pre-defined groups. Supervisor’s code allows same functions as user’s code plus the ability to change passwords, define groups and change reset time period. Touch Pad also has a hard coded manufacturer’s backdoor password which can be provided by manufacturer in case passwords are compromised or forgotten. Touch Pad automatically reverts back to secure mode if inactive for more than the user defined reset time period. Multiple Touch pads (up to 16) can be used on one any system.

D. Groups shall be definable by user with supervisor password. Groups are stored in each Touch Pad and each touch cad can support up to 10 groups with a maximum of eight devices per group. A maximum of four devices per relay/controller box can be included in a single group. Touch Pad includes logic to prevent the creation of groups that create a circuit overload.
E. Complete control system to include a combination of Model 503058 Relay/Controller Panels and Model 503057 Touch Pads.



F.
ELECTRICAL CONTRACTOR SHALL PROVIDE AND INSTALL: All conduits, wires (including low voltage), touch pad mounting box and programming.
*****Include the following paragraphs for practice cages with Smart Gym Group Control System.*****

A. Provide Smart Gym Group Controller capable of controlling Electric Divider Curtains, Electric Height Adjusters, Basketball Backstops with Electric Winches, Wrestling Mat Lifters and other Auxiliary Devices from a centralized location. Group Control Systems to have 120V control processor that is fully programmable to operate gymnasium equipment and auxiliary devices provided by others. Group Control System must be capable of operating up to 1000 devices and can be programmed to operate single devices or groups of up to four devices simultaneously.
B. Devices are controlled via 120V relay panels that may be mounted at remote location(s.) Each relay panel includes 8 relays. Devices that operate in an up/down or in/out cycle require two relays; devices that operate in an on/off cycle require one relay. Communications between processor and the relay panel(s) is 24V.

C. User interacts via a flush mounted graphical color Ethernet touch screen to be custom programmed to match the project conditions. Touch screen to require entry of four-digit security code prior to accessing control screens. Security code is to be fully programmable by the user. Touch screen should be mounted in full view of equipment being operated and a minimum of one touch screen should be each room with equipment being operated.

D. Complete control system to include one pre-programmed Model 503024 control processor and power supply in 18" x 18" x 6" surface mounted enclosure, the required number of Model 503027 relay panels in surface mounted 15" x 8" x 2" surface mounted enclosure and the requested number of Model 503038 color graphical touch screen to be mounted in transverse mounted standard single-gang electrical switch box.



E.
ELECTRICAL CONTRACTOR SHALL PROVIDE AND INSTALL: All conduits, wiring (including low voltage and Ethernet) touch screen electrical switch box.

PART 3 - EXECUTION
3.1

seq level1 \h \r0 

seq level2 \h \r0 
PREPARATION
*****Typically gymnasium practice cages are suspended from beam or rigid frame flanges with beam clamps, brackets, and threaded rods provided by Progressive Sports Construction Group. If required, supplementary beams, bracing, and other support members will need to be designed, detailed on drawings, and specified in other sections.*****



A.
Coordinate support of practice cage with roof structure to ensure proper distribution of loads and adequacy of attachment points. Ensure that building structure has been designed for loads of practice cage. Provide additional structural framing members as required in accordance with Section [_____] - [__________].


B.
Coordinate configuration, size, and installation of practice cage with height, slope, and type of building structure and lighting fixtures, mechanical equipment, ductwork, fire-suppression system, bleachers, athletic equipment, and other potential obstructions.



C.
Field verify dimensions prior to fabrication.



D.
Coordinate electrical requirements for motorized operating mechanism to ensure proper power source, conduit, wiring, and boxes for keyed switches. Prior to installation, verify type and location of power supply.



E.
For installations made after wood gymnasium flooring is installed, provide protection and exercise care not to damage flooring.

3.2

seq level1 \h \r0 

seq level2 \h \r0 
INSTALLATION



A.
Install in accordance with manufacturer's written instructions and shop drawings.



B.
Install even and level with netting hanging straight in down position.



C.
Install control switch such that operator has view of complete practice cage during lowering and raising.



D.
Adjust limit switches of electric winch to ensure accurate position in both stored and lowered positions.

3.3

seq level1 \h \r0 

seq level2 \h \r0 
TESTING AND DEMONSTRATION



A.
Operate practice cage to ensure proper lifting and lowering. Adjust as required to ensure smooth operation and accurate positioning.



B.
Demonstrate to Owner's designated representatives complete operation and required maintenance.
END OF SECTION
______________________________________________________________________________

GYMNASIUM PRACTICE CAGES
11 66 53.13- 7

