PROGRESSIVE SPORTS CONSTRUCTION GROUP PRODUCT SPECIFICATIONS     
July 2014
Progressive Sports Construction Group, 411 South Pearl Street, Spiceland, Indiana U.S.A. 47385-0425

TEL:  877-413-0957;  WEB SITE:  www.ProgressiveSCG.com


SECTION 11 66 53 

GYMNASIUM DIVIDERS

*****  Progressive Sports Construction Group manufactures the following manual and motorized fabric gymnasium dividers to subdivide gymnasiums and other recreational rooms into two areas:



Walk-Draw Gym Divider



Roll-Up Gym Divider



Fold-Up Gym Divider


Top-Roll Gym Divider


Ridge-Fold Gym Divider
For each Progressive Sports Construction Group gymnasium divider there are options for curtain material, color, size, configuration, operation, and method of installation.  Refer to Progressive Sports Construction Group product literature for availability of options, standard colors, and guidelines for design, selection, and installation of divider.  Various models and options are listed in this specification section, but not all options apply to all gymnasium dividers.  The specifier will need to select the appropriate options and delete non-applicable models and options.  

PART 1 - GENERAL
1.1
SUMMARY


A.
Section includes:  [Manually] [Electrically] operated fabric gymnasium divider.



B.
Related sections:

*****  List other specification sections dealing with work directly related to this section such as the following.  *****



1.
Section [_____] - [__________]:  Structural steel framing to support gymnasium divider.




2.
Section 16[___] - [__________]:  Electrical supply, conduit, and wiring for motorized gymnasium divider.

1.2

seq level1 \h \r0 

seq level2 \h \r0 

seq level3 \h \r0 
SUBMITTALS


A.
Submit in accordance with Section 01 33 00 [01330] - Submittal Procedures:




1.
List of proposed products and product data.




2.
Loads to be transmitted to building structural members and requirements for supplementary bracing and structural support members.




3.
Shop drawings showing layout, elevations, dimensions, fabrication details, method of attachment and electrical wiring diagrams.




4.
Manufacturer must provide calculations and reports for tests preformed by an independent testing laboratory accredited by the American Association of Laboratory Accreditation (A2LA) that clearly demonstrate compliance with minimum safety factors included in product specifications.



5.
Certificates for Divider Curtain Vinyl and Mesh to prove they meet the requirements of Greenguard Gold



6.
Samples of fabric [for selection by Architect].




7.
Manufacturer's installation and maintenance instructions.

1.3

seq level1 \h \r0 

seq level2 \h \r0 

seq level3 \h \r0 
QUALITY ASSURANCE


A.
Source limitation:  All components including curtain, suspension system, electric winches, and controls for divider shall be products of a single manufacturer.



B.
All welding to be performed by personnel having passed Welder Qualification testing in accordance with American Welding Society (AWS) code D1.1 or higher. Manufacturer to provide certification and test results upon request
1.4

seq level1 \h \r0 

seq level2 \h \r0 
DELIVERY, STORAGE, AND HANDLING
*****  In order to reduce potential damage to curtain, it is desirable to deliver and install dividers after gymnasium flooring and other interior construction is essentially complete.  *****


A.
Do not deliver divider until building is enclosed and other construction within gymnasium is substantially complete.

PARTS 2 - PRODUCTS
2.1

seq level1 \h \r0 

seq level2 \h \r0 
ACCEPTABLE MANUFACTURERS


A.
Progressive Sports Construction Group, 411 South Pearl Street, Spiceland, Indiana 47385-0425; 765-987-7999.



B.
Manufacturers of equivalent products submitted and approved in accordance with Section 00 26 00 [01630] - Product Substitution Procedures.

2.2

seq level1 \h \r0 

seq level2 \h \r0 
GYMNASIUM DIVIDER
*****  Select the type and model of gymnasium divider being specified and appropriate attributes from the following options.  *****

*****  For Progressive Sports Construction Group WALK-DRAW GYM DIVIDER, use the following paragraphs.  ****


A.
Type:  Manually operated, horizontal pull type gymnasium divider including clamps for attachment to building structure, and other components required for complete functional installation; Walk-Draw Gym Divider as manufactured by Progressive Sports Construction Group Company, Inc.



B.
Operation:  Curtain suspended from overhead track and manually pulled open and closed while walking.

*****  Progressive Sports Construction Group Walk-Draw Gym Divider can be custom fabricated to accommodate various shapes with single or multiple curtains and straight or curved tracks.  Curtains may be bi-parting or stack on one side.  Maximum radius for curved tracks is 2 feet (610 mm).  *****


C.
Configuration:  [Single] [2] [3] [4] [_____] curtain[s] arranged in [straight line] [curved] [box] [_____] shape as indicated on Drawings.  [Curtains to be bi-parting with [[3 feet] [914 mm] overlap at opening.] [velcro fasteners at opening.]] [Curtain to stack on one side.]  




1.
Stack space for curtain shall not exceed [1-1/8 inches] [29 mm] per carrier.




2.
Minimum required clearance for curtain on each side of track:  [6 inches] [152 mm].



D.seq level3 \h \r0 
Track:  16 gage galvanized steel channel-type track [1-1/2 by 1-1/4 inches] [38 by 32 mm] attached to structural support [with beam clamps and hanger brackets] [__________] at [8 feet] [2.4 m] maximum on center.



E.
Attachment:  Attachment:  Attach to structural support with beam clamps, hanger brackets, and [1/2 inch] [13 mm] diameter threaded rods. Attachment clamps designed to be capable of supporting a minimum of 5,000 lbs each and provided in sufficient number to provide a combined minimum 45:1 attachment point safety factor.


F.
Carriers:  [1-1/8 inches] [29 mm] diameter ball bearing wheels with trim chain for attachment to grommets in curtain top.  Space carriers at [12 inches] [305 mm].



G.
Divider bottom:  Weighted with No. 2/0 coil proof chain in curtain pocket.

******************************************************************************
*****  For Progressive Sports Construction Group ROLL-UP GYM DIVIDER, use the following paragraphs.  ********


A.
Type:  Electrically operated, roll-up gymnasium divider including motor, belts, controls, clamps for attachment to building structure, threaded rod supports, and other components required for complete functional installation; Roll-Up Gym Divider as manufactured by Progressive Sports Construction Group.



B.
Operation:  Curtain rolled up and down by belts wound onto overhead rotating drive pipe operated by electrical motor.

*****  Progressive Sports Construction Group Roll-Up Gym Divider can be custom fabricated to any size but cannot be cutout for bleachers or other objects.  It is recommended that space be provided for people to walk around ends of divider.  *****


C.
Configuration:  Rectangular shape with straight bottom and extending across room as indicated on Drawings.




1.
Maximum dimension of stored divider:  [2 feet] [610 mm] from bottom of structural support to bottom of rolled curtain.




2.
Minimum required clearance between vertical curtain edges and adjacent fixed objects:  [6 inches] [152 mm].




3.
Provide [36 inches] [1 m] [_____] space between curtain ends and walls or fixed objects to allow passage space around divider.



D.
Operating mechanism:  Drive pipe winch powered with 3/4 HP, 110VAC, 60-cycle, single-phase, reversible capacitor, C-Face motor with thermal overload protection.  Entire winch assembly shall carry a five-year warranty.  Provide with load holding worm gear reduction and integral limit switches to control curtain travel.  Drive pipe shall rotate in pipe support assemblies spaced at approximately [9 feet] [2.7 m].



E.
Attachment:  Attach to structural support with beam clamps, hanger brackets, and [1/2 inch] [13 mm] diameter threaded rods. Attachment clamps designed to be capable of supporting a minimum of 5,000 lbs each and provided in sufficient number to provide a combined minimum 45:1 attachment point safety factor. 


F.
Hoist belts:  [5 inches] [127 mm] wide white polyester webbing attached to drive pipe, passing under bottom batten, and terminating at top batten.  Space belts at approximately [15 feet] [4.6 m]. 



G.
Bottom roller:  [3-1/2 inches] [89 mm] diameter steel pipe with aluminum strip for attachment of curtain.

******************************************************************************

*****  For Progressive Sports Construction Group FOLD-UP GYM DIVIDER, use the following paragraphs.  ********


A.
Type:  Electrically operated, fold-up gymnasium divider including motor, cables, controls, clamps for attachment to building structure, threaded rod supports, and other components required for complete functional installation; Fold-Up Gym Divider as manufactured by Progressive Sports Construction Group.



B.
Operation:  Curtain moves by accordion fold-up action as bottom steel pipe is raised by hoist lines passing through grommets.

*****  Progressive Sports Construction Group Fold-Up Gym Divider can be custom fabricated to any size but cannot be cutout for bleachers or other objects.  It is recommended that space be provided for people to walk around ends of divider.  *****


C.
Configuration:  Rectangular shape with straight bottom and extending across room as indicated on Drawings.




1.
Stored divider dimensions from bottom of structural support to bottom of folded curtain:  





a.
Maximum:  [42 inches] [1067 mm].





b.
Minimum:  [30 inches] [762 mm].




2.
Minimum required clearance between vertical curtain edges and adjacent fixed objects:  [6 inches] [152 mm].




3.
Provide [36 inches] [1 m] [_____] space between curtain ends and walls or fixed objects to allow passage space around divider.



D.
Operating mechanism:  Drive pipe winch powered with 3/4 HP, 110VAC, 60-cycle, single-phase, reversible capacitor, C-Face motor with thermal overload protection.  Entire winch assembly shall carry a five-year warranty.  Provide with load holding worm gear reduction and integral limit switches to control curtain travel.  Drive pipe shall rotate in pipe support assemblies spaced at approximately [9 feet] [2.7 m].



E.
Attachment:  Attach to structural support with beam clamps, hanger brackets, and [1/2 inch] [13 mm] diameter threaded rods. Attachment clamps designed to be capable of supporting a minimum of 5,000 lbs each and provided in sufficient number to provide a combined minimum 45:1 attachment point safety factor.


F.
Hoist lines:  [1/8 inch] [3 mm] diameter steel cable with 2,000 pounds minimum breaking strength attached to bottom batten and passing through curtain grommets at 18 inches to terminate at top drive pipe.  Space lines at approximately [111 inches] [2819 mm]. 



G.
Divider bottom:  Hoist lines secured to [1-5/8 inches] [41 mm] diameter steel pipe batten in [6 inches] [152 mm] wide curtain pocket.

******************************************************************************
*****  For Progressive Sports Construction Group TOP-ROLL GYM DIVIDER, use the following paragraphs.  ****
A.
Type:  Electrically operated, top-roll gymnasium divider including motor, controls, clamps for attachment to building structure, threaded rod supports, and other components required for complete functional installation; Top-Roll Gym Divider as manufactured by Progressive Sports Construction Group.



B.
Operation:  Curtain moves by rolling directly onto drive tube without the use of belts or cables.
*****  Progressive Sports Construction Group Top-Roll Gym Divider can be custom fabricated to any size but cannot be cutout for bleachers or other objects.  It is recommended that space be provided for people to walk around ends of divider.  *****


C.
Configuration:  Rectangular shape with straight bottom and extending across room as indicated on Drawings.




1.
Maximum dimension of stored divider:  [14 inches] [355 mm] from bottom of structural support to bottom of rolled curtain.




2.
Minimum required clearance between vertical curtain edges and adjacent fixed objects:  [6 inches] [152 mm].




3.
Provide [36 inches] [1 m] [_____] space between curtain ends and walls or fixed objects to allow passage space around divider.



D.
Operating mechanism:  Drive pipe winch powered with 3/4 HP, 110VAC, 60-cycle, single-phase, reversible capacitor, C-Face motor with thermal overload protection.  Entire winch assembly shall carry a five-year warranty.  Provide with load holding worm gear reduction and integral limit switches to control curtain travel.  Drive pipe shall rotate in pipe support assemblies spaced at approximately [10 feet] [3.1 m].



E.
Attachment:  Attach to structural support with beam clamps, hanger brackets, and [1/2 inch] [13 mm] diameter threaded rods. Attachment clamps designed to be capable of supporting a minimum of 5,000 lbs each and provided in sufficient number to provide a combined minimum 45:1 attachment point safety factor.


F.
Drive pipe:  [5 inches] [127 mm] diameter steel pipe.  Drive pipe shall roll in precision laser cut and formed support assemblies.  Assemblies spaced at a maximum of 10 feet [3 m] on center 



G.
Divider bottom:  [1-5/8 inches] [41 mm] diameter steel pipe batten in 6 inches [152 mm] wide curtain pocket.

******************************************************************************
*****  For Progressive Sports Construction Group RIDGE-FOLD GYM DIVIDER, use the following paragraphs.  ****


A.
Type:  Electrically operated, fold-up gymnasium divider conforming to roof slope and including motor winch, cables, sheave assemblies, controls, clamps for attachment to building structure, supports, and other components required for complete functional installation; Ridge-Fold Gym Divider as manufactured by Progressive Sports Construction Group Company, Inc.



B.
Operation:  Gym divider shall utilize pivot joints to fold compactly into roof contour.  Curtain moves by accordion fold-up action as weighted bottom is raised by hoist lines passing through curtain grommets.  Hoist lines pass through sheave assemblies attached to roof structure.  Lines pass through intermediate sheaves which act as idler assemblies and direct lines to motorized winch.  Lines terminate at individual drums sized in variable ratios allowing raised curtain to conform to roof slope.

*****  Progressive Sports Construction Group Ridge-Fold Gym Divider can be custom fabricated to any size but cannot be cutout for bleachers or other objects.  It is recommended that space be provided for people to walk around ends of divider.  *****


C.
Configuration:  Pivot joints in top and bottom batten pipes allow divider to conform to roof contour in both open and stored positions.  Divider extends across room as indicated on Drawings.




1.
Minimum required clearance between vertical curtain edges and adjacent fixed objects:  [6 inches] [152 mm].




2.
Provide [36 inches] [1 m] [_____] space between curtain ends and walls or fixed objects to allow passage space around divider.



D.
Operating winch:  Powered with C-Face motor sized for specific project torque requirements and equipped with thermal overload protection and rotary counting limit switch to control curtain travel.  Individual drums sized in variable ratios shall be provided for factory termination of each cable.  Winch to be attached to roof structure.



E.
Sheaves:  Heavy-duty assemblies with roller bearings and attached to roof structure at each hoist location.



F.
Hoist lines:  [1/8 inch] [3 mm] diameter steel cable with 2,100 pounds minimum breaking strength.  Lines shall attach to battens concealed at curtain bottom, pass through curtain grommets at [18 inches] [547 mm] to sheave assembly, continue through intermediate sheaves along slope of roof to winch, and terminate at individual drums.  Space lines at approximately [111 inches] [2819 mm]. 



G.
Top curtain support:  [1-5/8 inches] [41 mm] diameter steel pipes with pivot joints in curtain pocket and suspended from sheave assembly and roof structure with No. 2/0 chain.



E.
Attachment:  Attach sheaves and winch to structural support with beam clamps, steel angle brackets.  



G.
Divider bottom:  Hoist lines secured to [1-5/8 inches] [41 mm] diameter steel pipe batten with pivot joints in [6 inches] [152 mm] wide curtain pocket.

******************************************************************************

*****  For Progressive Sports Construction Group PERIMETER GYM DIVIDER, use the following paragraphs.  ********


A.
Type:  Electrically operated, perimeter gymnasium divider including motor, cables, controls, clamps for attachment to building structure, threaded rod supports, and other components required for complete functional installation; Permiter Gym Divider as manufactured by Progressive Sports Construction Group.



B.
Operation:  Curtain moves by accordion fold-up action as bottom steel pipe is raised by hoist lines passing through grommets.

*****  Progressive Sports Construction Group Permiter Gym Divider can be custom fabricated to any size but cannot be cutout for bleachers or other objects.  It is recommended that space be provided for people to walk around ends of divider.  *****


C.
Configuration:  Rectangular shape fabric that has segmented straight bottom bottom section joined at angles to approximate desired radius as indicated on Drawings.




1.
Stored divider dimensions from bottom of structural support to bottom of folded curtain:  





a.
Maximum:  [42 inches] [1067 mm].





b.
Minimum:  [30 inches] [762 mm].




2.
Minimum required clearance between vertical curtain edges and adjacent fixed objects:  [6 inches] [152 mm].




3.
Provide [36 inches] [1 m] [_____] space between curtain ends and walls or fixed objects to allow passage space around divider.



D.
Operating mechanism:  Drive pipe winch powered with 3/4 HP, 110VAC, 60-cycle, single-phase, reversible capacitor, C-Face motor with thermal overload protection.  Entire winch assembly shall carry a five-year warranty.  Provide with load holding worm gear reduction and integral limit switches to control curtain travel.  Drive pipe shall rotate in pipe support assemblies spaced at approximately [9 feet] [2.7 m].



E.
Attachment:  Attach to structural support with beam clamps, hanger brackets, and [1/2 inch] [13 mm] diameter threaded rods. Attachment clamps designed to be capable of supporting a minimum of 5,000 lbs each and provided in sufficient number to provide a combined minimum 45:1 attachment point safety factor.



F.
Hoist lines:  [1/8 inch] [3 mm] diameter steel cable with 2,000 pounds minimum breaking strength attached to bottom batten and passing through curtain grommets at 18 inches to terminate at top drive pipe.  Space lines at approximately [111 inches] [2819 mm]. 



G.
Divider bottom:  Hoist lines secured to [1-5/8 inches] [41 mm] diameter steel pipe batten in [6 inches] [152 mm] wide curtain pocket.

******************************************************************************
2.3

seq level1 \h \r0 

seq level2 \h \r0 

seq level3 \h \r0 

seq level4 \h \r0 
CURTAIN

*****  Curtains for Progressive Sports Construction Group gymnasium dividers are fabricated from vinyl coated polyester mesh, solid vinyl coated polyester, or a combination of solid fabric and mesh. 


A.
[Full height curtain] [Bottom [8 feet] [2.4 m] [_____] feet]:  Opaque solid vinyl coated polyester fabric:

*****  Standard solid fabric weight is 18 ounces per SY and is available in eleven colors.  Fabric weighing 22 ounces per SY is available in red, dark blue and grey colors ON ALL CURTAINS EXCEPT Top Roll Dividers:  Only 18 oz is available for Top Roll.  *****



1.
Weight:  [18] [22] ounces per SY.




2.
Resistant to rot, mildew, and ultraviolet light.




3.
Flammability:  Rated self extinguishing in accordance with California State Fire Code F-31.5 and F-140.



4.
Color:  [White] [Red] [Yellow] [Dark blue] [Marine blue] [Beige] [Grey] [Black] [Forest green] [Orange] [Maroon] [Selected by Architect from manufacturer's standard range.]



B.seq level3 \h \r0 
[Full height curtain] [Upper curtain section]:  Vinyl coated polyester mesh.




1.
Weight:  9 ounces per SY.




2.
Resistant to rot, mildew, and ultraviolet light.




3.
Flammability:  Rated self extinguishing in accordance with California State Fire Code F-230.




4.
Color:  [White] [Red] [Yellow] [Blue and black weave] [Grey] [Black] [Selected by Architect from manufacturer's standard range.]



C.
VOC Emission:  Divider Curtain Vinyl and Mesh to be low emitting and certified to meet all of the requirements of the GREENGUARD Gold and GREENGUARD certification program.  GREENGUARD Gold  requires emissions of total volatile organic compounds ≤ 0.22 mg/m3, formaldehyde ≤ 0.0135 ppm, total aldehydes ≤ 0.043 ppm, individual volatile organic compounds ≤ 1/1000 TLV and ≤ ½ chronic REL and total phthalates ≤ 0.01 mg/m3. Vinyl and Mesh must be evaluated to indoor air quality evaluation (IAQ) using a GREENGUARD product evaluation protocol following the requirements of UL Environment Product Certification Program, ASTM Standard D5116 and the United States Environmental Protection Agency and modeled based on UL requirements for a standard gymnasium loading and ASHRAE 62.1 – 2004 ventilation conditions.  Manufacturer to provide certificate and/or test results upon request.
*****  Seams for Progressive Sports Construction Group Walk-Draw, Fold-Up, Permiter and Ridge-Fold are vertical.  Seams for Roll-Up and Top-Roll are horizontal.


D.seq level3 \h \r0 
Seams:  [Vertical] [Horizontal] and electronically welded with [1 inch] [25 mm] full contact weld.



E.
Outer edge hems:  Triple turned with double welds.

*****  For Walk-Draw curtains use the following paragraphs.  *********************



F.
Top edge:  Solid fabric in double thickness and double welded to [mesh] [solid curtain fabric] to form [3 inches] [76 mm] wide top hem with grommets spaced at [12 inches] [304 mm].



G.
Bottom edge:  Pocket to house chain weights.

******************************************************************************

*****  For Roll-Up curtains use following paragraphs.  ****************************



F.
Top edge:  Solid fabric in triple thickness and double welded to [mesh] [solid curtain fabric] to form [6 inches] [152 mm] wide pocket for top pipe batten.



G.
Bottom edge cut square for attachment to roller pipe with aluminum stop strip.

******************************************************************************

*****  For Top-Roll curtains use following paragraphs.  ****************************



F.
Top edge:  Solid fabric, cut square for attachment to roller tube and of sufficient length to allow at least two complete wraps on roller tube at all times.



G.
Bottom edge:  [Padded] Pocket to house bottom pipe batten.

*****  For Fold-Up, Perimeter and Ridge-Fold curtains use the following paragraphs.  ***********



F.
Top edge:  Solid fabric in triple thickness and double welded to [mesh] [solid curtain fabric] to form [6 inches] [152 mm] wide pocket for top pipe batten.



G.
Bottom edge:  [Padded] Pocket to house bottom pipe batten.

******************************************************************************

2.4

seq level1 \h \r0 

seq level2 \h \r0 
CURTAIN SAFETY DEVICE

*****  Include the following paragraphs for Roll-Up, Top-Roll, Perimeter and Fold-Up Progressive Sports Construction Group gymnasium dividers to include Curtain Lok safety device *****

*****************************************************************************

A. Provide Progressive Sports Construction Group Model 504301 Curtain Lok safety device. Curtain Lok to be directly speed sensitive to automatically lock divider curtain in position at any time during storage or operation.  In the event of an over-speed situation (greater than 1.5 feet per second) caused by malfunction of the hoisting apparatus, whether sudden or gradual, device will immediately activate.  Curtain Lok work regardless of direction of rotation and automatically resets when load is reversed or removed.  

2.5seq level1 \h \r0 

seq level2 \h \r0 
CONTROLS

*****  Include the following paragraphs for Roll-Up, Top-Roll, Fold-Up, Perimeter and Ridge-Fold Progressive Sports Construction Group gymnasium dividers with standard control for motorized dividers, 3-position key lock wall switch.  *****



A.
Provide key lock, 3-position, momentary contact wall control switch to lower, raise, and stop gymnasium divider.  Provide with switch box and plastic cover plate.

*****  Include the following paragraphs for Roll-Up, Top-Roll, Fold-Up, Perimeter and Ridge-Fold Progressive Sports Construction Group gymnasium dividers with MEC Screen Group Control System.  *****

A.    Provide Mec Screen Group Controller capable of controlling Electric Divider Curtains, Electric Height Adjusters, Basketball Backstops with Electric Winches, Wrestling Mat Lifters and other Auxiliary Devices from a centralized location.  Group Control Systems to have 120V control processor that is fully programmable to operate gymnasium equipment and auxiliary devices provided by others.  Group Control System must be capable of operating up to 1000 devices and can be programmed to operate single devices or groups of up to four devices simultaneously. 

B.    Devices are controlled via 120V relay panels that may be mounted at remote location(s.) Each relay panel includes 8 relays.  Devices that operate in an up/down or in/out cycle require two relays; devices that operate in an on/off cycle require one relay.  Communications between processor and the relay panel(s) is 24V.

C.   User interacts via a 24V flush mounted graphical color touch screen to be custom programmed to match the project conditions.  Touch screen to require entry of four-digit security code prior to accessing control screens.  Security code is to be fully programmable by the user. Touch screen should be mounted in full view of equipment being operated and a minimum of one touch screen should be each room with equipment being operated.   

D.   Complete control system to include one pre-programmed Model 503024 control processor and power supply in 18" x 18" x 6" surface mounted enclosure, the required number of Model 503027 relay panels in surface mounted 15" x 8" x 2" surface mounted enclosure and the requested number of Model 503038 color graphical ethernet touch screen to be mounted in a horizontally mounted  single gang electrical switch box.



E.
ELECTRICAL CONTRACTOR SHALL PROVIDE AND INSTALL: All conduits complete with wire from power source to control processor, from control processor to relay panels, from relay panels to devices and from control processor to touch screen and one single gang electrical switch box for mounting each touch screen. 

PART 3 - EXECUTION
3.1

seq level1 \h \r0 

seq level2 \h \r0 
PREPARATION
*****  Progressive Sports Construction Group gymnasium dividers can be suspended directly under continuous roof beam, between two beams, and at right angles to beams.  Typically dividers are suspended from beam or rigid frame flanges with beam clamps, brackets, and threaded rods provider by Progressive Sports Construction Group.  If required, supplementary beams, bracing, and other support members will need to be designed, detailed on drawings, and specified in other sections.  *****



A.
Coordinate support of gymnasium divider with roof structure to ensure proper distribution of loads and adequacy of attachment points.  Ensure that building structure has been designed for loads of specific gymnasium divider to be provided.  Provide additional structural framing members as required in accordance with Section [_____] - [__________].



B.
Coordinate configuration, size, and installation of gymnasium divider with height, slope, and type of building structure and lighting fixtures, mechanical equipment, ductwork, fire-suppression system, bleachers, athletic equipment, and other potential obstructions.



C.
Field verify dimensions prior to fabrication.

*****  Include the following paragraph for motorized gymnasium dividers.  *****


D.
Coordinate electrical requirements for motorized operating mechanism to ensure proper power source, conduit, wiring, and boxes for keyed switches.  Prior to installation, verify type and location of power supply.



E.
For installations made after wood gymnasium flooring is installed, provide protection and exercise care not damage flooring.

3.2

seq level1 \h \r0 

seq level2 \h \r0 
INSTALLATION



A.
Install in accordance with manufacturer's written instructions and shop drawings.



B.
Install even and level with curtain hanging [2 inches] [50 mm] [_____] above floor in down position.

*****  Include the following two paragraphs for motorized gymnasium dividers.  *****


C.
Install control switch such that operator has view of complete gymnasium divider during lowering and raising.



D.
Adjust limit switches of electric winch to ensure accurate position in both stored and lowered positions.

3.3

seq level1 \h \r0 

seq level2 \h \r0 
TESTING AND DEMONSTRATION



A.
Operate divider curtains to ensure proper lifting and lowering.  Adjust as required to ensure smooth operation and accurate positioning.



B.
Demonstrate to Owner's designated representatives complete operation and required maintenance.


END OF SECTION
______________________________________________________________________________

GYMNASIUM DIVIDERS
11484 - 1

